| FallCreek Laser Pattern Seating Part 1 General 1.1 Description Section includes information, materials, and options for products manufactured by SiteScapes, Inc. All specifications are subject to change. Contact manufacturer for details. 1.2 Submittals Specification Drawing: Detail drawing of product including overall dimensions and options. Samples: Various component samples available upon request. Qualifications: Installer must submit evidence of a successful installation history with comparable materials and designs specified. 1.3 Delivery, Storage, and Handling Delivery: Deliver products to site in manufacturer’s original, unopened containers and packaging. Upon delivery, examine packages immediately to ensure all products are complete and undamaged. Storage: Store products in a protected, dry area in original undamaged packaging and containers until ready for installation. Handling: Handle powder coated products with sufficient care to prevent any scratches or damage to the finish. 1.4 Warranty Powder coat: Powder coat products are warranted against defects in material or workmanship for a period of one year. DuraCoat: DuraCoat products are warranted against defects in material or workmanship for a period of five years. Stainless Steel / Galvanized: Stainless steel and galvanized products are warranted against defects in material or workmanship for a period of five years. FallCreek Laser Pattern Seating Part 2 Products 2.1 Product Name WestPor Horizontal Strap Seating 2.2 Manufacturer SiteScapes Inc. P.O. Box 22326 Lincoln, NE 68542 Phone: 888.331.9464 Fax: 402.421.9479 www.sitescapesonline.com info@sitescapesonline.com 2.3 Materials Frame - 1 1/2" Sq. x 11 Ga. Wall Steel Tubing Panels - 7 Ga. Steel Sheet Metal Foot Plates: Foot plates are 5/16” x 1 1/2" stainless steel. Each plate has a 9/16” hole for mounting. Mounting: Benches are secured with four 1/2" dia. x 4-5” long stainless steel anchor bolts, which are customer supplied. 2.4 Options Length: Available lengths are 6’ (72”L x 34”h x 28”d) and 8’ (96”L x 34”h x 28”d). Back: FallCreek benches are offered in backed and backless varieties. FallCreek Laser Pattern Seating Part 3 Execution 3.1 Assembly FallCreek benches are shipped fully assembled. 3.2 Installation Benches may be surface mounted using the holes that are predrilled in each of the footplates. Benches are to be mounted with four Ø1/2” x 4-5” stainless steel anchor bolts, which are customer supplied. It is not recommended to locate anchor bolts until product is in place. This SiteScapes, Inc. product must be permanently affixed to the ground. Consult your local codes for regulations. 3.3 Maintenance To keep the finish on your products clean we suggest washing them once or twice monthly with any multi-purpose cleaner. For products with excess dirt or grime we suggest washing the product with a 2/1 water/bleach mixture. In cases of severe scratching or vandalism, SiteScapes, Inc. offers touch up paint. This paint is an exact formulated match and is available in a standard liquid form or a spray can applicator. Additional charges may apply. 3.4 Protection Protect products prior to installation by having them remain in the manufacturer’s packaging and container. ADDITIONAL INFORMATION: For additional information or questions please call SiteScapes toll free at 888.331.9464 between the hours of 8:00 a.m. and 5:00 p.m. C.S.T. END OF SPECIFICATIONS *Specifications are subject to change without notice. | Limited twenty year warranty against structural failure of steel bench frames or steel bench assemblies, table frames, trash receptacle frames, and planter frames. Limited 10 year breakage warranty on all ductile cast iron bench ends and components. SiteScapes, Inc. liability and the buyer’s remedy under these warranties are limited to the repair, replacement or refund of the purchase price, at SiteScapes option, of products determined in SiteScapes reasonable judgment to be defective in workmanship or materials. SiteScapes will not be held responsible for the costs incurred in labor to remove, install, or replace the alleged defective product. SiteScapes warranties do not cover any defects or failures resulting from vandalism, negligence, accidents, abuse, lack of maintenance, improper use or installation, or unapproved modifications and repairs. In addition, SiteScapes will not be held liable for ANY damage or injury as a result of product that is not permanently anchored in place or not anchored properly. 2.5 Finishes Powdercoat: All products are steel shot blasted, phosphatized, preheated and then coated with a TGIC exterior rated powder in the color of your choice. All parts are finished to an average of 8 to 10 mil thickness and fully cured to the manufactures specifications. Duracoat: All products are steel shot blasted, phosphatized, and preheated. Then each product is coated with a corrosion resistant, zinc rich epoxy undercoat. The TGIC exterior rated topcoat in the color of your choice is then applied. All parts are finished to an average of 8 to 10 mil thickness and fully cured to the manufactures specifications. Standard Colors: SiteScapes standard colors include: Onyx (Black) Pearl (White) Forest (Green) Cobalt (Dark Blue) Ivory (Ivory) Sterling (Silver) Slate (Dark Gray) Bronze (Brown) Brandywine (Burgundy). Custom colors are available at additional prices. Powder Coating Properties: Gloss at 60˚, ASTM D523-89: 80-90 Pencil Hardness, ASTM D3363-92a: 2H Dir / Rev Impact, Gardner, ASTM D2794-93: Up to 160 in/lbs Flexibility, Mandrel, ASTM D522-93a: ¼” dia., no fracture |
| --- | --- |